1.2. Design and Analysis of a Butterworth IIR High-Pass Filter Using MATLAB.

CODING:

clc;

clear;

close all;

% Specifications

Fs = 10000;      % Sampling Frequency

Fp = 3000;       % Passband Frequency

Fst = 2000;      % Stopband Frequency

Rp = 1;          % Passband Ripple (dB)

Rs = 60;         % Stopband Attenuation (dB)

% Butterworth High-Pass Filter Design

[N,Wn] = buttord(Fp/(Fs/2),Fst/(Fs/2),Rp,Rs);

[b,a] = butter(N,Wn,'high');

% Magnitude & Phase Response

figure;

freqz(b,a);

title('Magnitude and Phase Response');

% Impulse Response

figure;

[h,n] = impz(b,a,50);

stem(n,h,'filled');

grid on;

title('Impulse Response');

xlabel('Samples');

ylabel('Amplitude');

% Step Response

figure;

x = ones(1,50);          % Unit step input

y = filter(b,a,x);       % Step response

stem(0:49,y,'filled');

grid on;

title('Step Response');

xlabel('Samples');

ylabel('Amplitude');

% Group Delay

figure;

grpdelay(b,a);

title('Group Delay');

% Pole-Zero Plot

figure;

zplane(b,a);

title('Pole-Zero Plot');

% Display Results

fprintf('Minimum Filter Order (N) = %d\n', N);

fprintf('Cutoff Frequency (Wn) = %.4f\n', Wn);

OUTPUT: 1

OUTPUT: 2

OUTPUT: 3

OUTPUT: 4

OUTPUT: 5

RESULT:

Minimum Filter Order (N) = 12
        Cutoff Frequency (Wn) = 0.5807