6.1 Uniform Quantization and Quantization Error Analysis of a Discrete-Time  Sinusoidal Signal Using MATLAB

CODING:

clc;

clear;

close all;

%% Signal

fs = 1000;              % Sampling frequency

f = 50;                 % Signal frequency

t = 0:1/fs:0.1;         % Time vector

x = sin(2*pi*f*t);      % Original sine wave

%% Quantization Levels

L = [8 16 32];

colors = {'r','g','m'};

% Preallocate

xq = cell(1,length(L));

e = cell(1,length(L));

mse = zeros(1,length(L));

%% Plot Original and Quantized Signals

figure;

% Original Signal

subplot(4,1,1);

plot(t,x,'b','LineWidth',1.5);

title('Original Signal');

xlabel('Time (s)');

ylabel('Amplitude');

grid on;

for i = 1:length(L)

% Quantization

xq{i} = round(((x + 1) .* (L(i)-1)) ./ 2);

% Convert back to signal range [-1,1]

xq{i} = (xq{i} .* 2 ./ (L(i)-1)) - 1;

% Quantization Error

e{i} = x - xq{i};

% Mean Square Error

mse(i) = mean(e{i}.^2);

% Plot Quantized Signal

subplot(4,1,i+1);

stairs(t,xq{i},colors{i},'LineWidth',1.2);

title(['Quantized Signal (' num2str(L(i)) ' Levels)']);

xlabel('Time (s)');

ylabel('Amplitude');

grid on;

end

%% Display MSE

fprintf('\nMean Square Error (MSE):\n');

fprintf('MSE for 8 Levels  = %.6f\n', mse(1));

fprintf('MSE for 16 Levels = %.6f\n', mse(2));

fprintf('MSE for 32 Levels = %.6f\n', mse(3));

%% Error Plots

figure;

for i = 1:length(L)

subplot(3,1,i);

plot(t,e{i},'LineWidth',1.2);

title(['Quantization Error (' num2str(L(i)) ' Levels)']);

xlabel('Time (s)');

ylabel('Error');

grid on;

end

OUTPUT:

RESULT:

Mean Square Error (MSE):
MSE for 8 Levels  = 0.010473
MSE for 16 Levels = 0.001763
MSE for 32 Levels = 0.000357

>>