6.2 Analysis of Finite Word Length Effects in Digital Signal Representation Using

Truncation and Rounding Techniques in MATLAB

CODING:

clc;

clear;

close all;

%% Generate Signal

n = 0:79;

x = sin(2*pi*n/40);

%% Quantization

q = 8;

% Truncation quantization

xt = floor(x*q)/q;

% Rounding quantization

xr = round(x*q)/q;

%% Quantization Error

et = x - xt;       % Truncation error

er = x - xr;       % Rounding error

%% Figure 1: Original vs Quantized

figure;

plot(n, x, 'b', 'LineWidth', 1.5);

hold on;

plot(n, xr, 'r', 'LineWidth', 1.5);

grid on;

title('Original vs Quantized');

xlabel('Sample Number');

ylabel('Amplitude');

legend('Original', 'Quantized');

hold off;

%% Figure 2: Truncation vs Rounding

figure;

plot(n, x, 'b', 'LineWidth', 1.5);

hold on;

plot(n, xt, 'r', 'LineWidth', 1.5);

plot(n, xr, 'g', 'LineWidth', 1.5);

grid on;

title('Truncation vs Rounding');

xlabel('Sample Number');

ylabel('Amplitude');

legend('Original', 'Truncated', 'Rounded');

hold off;

%% Figure 3: Quantization Error

figure;

stem(n, et, 'r', 'filled');

hold on;

stem(n, er, 'g', 'filled');

grid on;

title('Quantization Error');

xlabel('Sample Number');

ylabel('Error');

legend('Truncation', 'Rounding');

hold off;

%% Figure 4: MSE Comparison

mse_truncation = mean(et.^2);

mse_rounding = mean(er.^2);

figure;

bar([mse_truncation, mse_rounding]);

set(gca, 'XTick', 1:2);

set(gca, 'XTickLabel', {'Truncation', 'Rounding'});

grid on;

title('MSE Comparison');

ylabel('MSE');

xlabel('Quantization Method');

%% Display MSE Values

fprintf('MSE for Truncation = %.6f\n', mse_truncation);

fprintf('MSE for Rounding   = %.6f\n', mse_rounding);

OUTPUT:

RESULT:

MSE for Truncation = 0.004426
MSE for Rounding   = 0.001610

>>