2.1  Design and performance Evaluation of a Chebyshev Type I IIR Low Pass Filter using MATLAB

CODING:

clc;

clear;

close all;

% Specifications

Rp = 1;      % Passband Ripple (dB)

As = 40;     % Stopband Attenuation (dB)

Wp = 0.4;    % Passband Edge

Ws = 0.6;    % Stopband Edge

[N,Wn]=cheb1ord(Wp,Ws,Rp,As);

[b,a]=cheby1(N,Rp,Wn);

fprintf('Filter Order = %d\n',N);

fprintf('Cutoff Frequency = %.2f * pi rad/sample\n',Wn);

if all(abs(roots(a))<1)

disp('System is STABLE');

else

disp('System is UNSTABLE');

end

% -------- GRAPHS --------

figure;

freqz(b,a);          % Magnitude + Phase

figure;

impz(b,a);           % Impulse Response

figure;

grpdelay(b,a);       % Group Delay

figure;

zplane(b,a);         % Pole-Zero

OUTPUT: 1

OUTPUT: 2

OUTPUT: 3

OUTPUT: 4

RESULT:

Filter Order = 5
Cutoff Frequency = 0.40 * pi rad/sample
System is STABLE